Methodology for estimation of land cover for hydrologic modeling in Arequipa, Peru

Fariborz Daneshvar1, Jane R. Frankenberger1, Laura C. Bowling2, Keith A. Cherkauer1, Johann A. Vera Mercado1

1Agricultural and Biological Engineering Department, Purdue University, West Lafayette, IN USA

2Agronomy Department, Purdue University, West Lafayette, IN USA

Table of Contents

1. Introduction:	2

2. Available Land Cover Data	3

3. SWAT Equivalent Land Cover	5

3.1. Cultivos Permanentes (Permanent crops)	6

3.2.  Áreas Urbanizadas (Urbanized Areas)	6

3.3. Vegetación Asociada a Cuerpos de Agua (Vegetation associated with water)	7

3.4. Áreas con Vegetación Herbácea y/o Arbustiva (Areas with herbaceous and/or shrubby vegetation)	9

3.5. Áreas Sin o Con Poca Vegetación (Areas without or with little vegetation)	13

3.6. Bosques (Forest)	13

3.7. Aguas continentals (Waterbodies)	15

4. Regionalization of Land Cover Parameters	16

4.1. Leaf Area Index Parameters	16

4.2. Optimal Temperature for Plant Growth	17

5. Sample Results	19

6. References:	21

Appendix 1. Land Cover/Plant Growth parameters needed for SWAT	23

Appendix 2. List of crops that are available in SWAT database [2]	25

# 1. Introduction:

Hydrologic models including the Soil and Water Assessment Tool (SWAT) require spatially distributed land cover data and associated crop type parameters. The database provided in the SWAT model includes properties for broad land cover categories based on observations in the U.S (listed in Appendix 2), but these may not capture the characteristics of similar categories around the world. This methodology has been developed to extend the SWAT land cover database and make it usable for SWAT simulation in the Arequipa Region of Peru.

A land cover map for the Arequipa Region is available that provides Spanish terms for each vegetation or other land cover type, but to use the map with SWAT, parameters have to be developed for the regional land cover types. The focus of SWAT simulations for the Arequipa Region was on hydrology, so only parameters related to water quantity were developed. Parameters that only affect water quality were kept as default for the base land cover category. This document explains how regional land cover database was created in two steps. Section 2 of this document explores the available land cover data, then in Section 3 equivalent SWAT land cover types were assigned to each original land cover type to serve as the basis for developing a local parameter set, and in Section 4, the parameters were modified to reflect how Peruvian land cover responds to local climate conditions, while Section 5 provides example of results for one land cover type.

# 2. Available Land Cover Data

A regional land cover map of Arequipa Region, created for the Ecological and Economic Zoning (Zonificación Ecológica y Económica or ZEE) conducted by the Ministry of Environment (Ministerio de Ambiente) for all regions of Peru [1] (Figure 1). It includes three levels of classification, as listed in Table 1.

Figure 1. ZEE map of land cover for the Arequipa Region (Gobierno Regional Arequipa, 2016)

Table 1. Land cover/ vegetation classification of the Arequipa Region [1] and (English translations in parentheses)

| Level I | Level II | Level III | Area (ha) | Area (%) |
| --- | --- | --- | --- | --- |
| Áreas Agrícolas (Agricultural areas) | Cultivos Permanentes (Permanent crops) | Cultivos Agrícolas (Agricultural crops) | 151,574 | 2.39 |
| Áreas Artificializadas (Artificialized areas) | Áreas Urbanizadas (Urbanized Areas) | Zonas Urbanas (Urban areas) | 20,417 | 0.32 |
| Áreas Húmedas (Wet areas) | Vegetación Asociada a Cuerpos de Agua (Vegetation associated with water) | Bofedales (Peat bogs) | 219,588 | 3.47 |
|  |  | Humedal Costero (Coastal Wetland) | 622 | 0.01 |
|  |  | Monte Ribereño (Floodplain) | 34,327 | 0.54 |
| Bosques y Áreas Mayormente Naturales (Forests and mostly natural areas) | Áreas con Vegetación Herbácea y/o Arbustiva (Areas with herbaceous and/or shrubby vegetation) | Césped de Puna (Puna grass) | 607,008 | 9.59 |
|  |  | Lomas (Fog-watered vegetation) | 332,021 | 5.24 |
|  |  | Matorral (Scrub) | 493,163 | 7.79 |
|  |  | Pajonal (Scrubland) | 507,972 | 8.02 |
|  |  | Tillandsia (Evergreen flowering plants) | 16,609 | 0.26 |
|  |  | Tolares (Resinous shrub) | 714,632 | 11.29 |
|  |  | Yaretales (High altitude shrubs) | 22,898 | 0.36 |
|  | Áreas Sin o Con Poca Vegetación (Areas without or with little vegetation) | Áreas con Vegetación Herbácea Veraniegas (Areas with summer herbaceous vegetation) | 445,338 | 7.03 |
|  |  | Sin Vegetación (No vegetation) | 2,234,653 | 35.29 |
|  |  | Vegetación de Suelos Crioturbados (Frozen soil vegetation) | 316,234 | 4.99 |
|  | Bosques (Forest) | Bosque de Lomas (Fog-watered forest) | 3,290 | 0.05 |
|  |  | Queñuales (Polylepis) | 63,837 | 1.01 |
|  |  | Cactaceas (Cactus) | 118,300 | 1.87 |
|  |  | Puyas (Giant bromeliad) | 8,526 | 0.13 |
| Superficies de Agua (Surface water) | Aguas continentales (Waterbodies) | Embalses (Reservoir) | 4,138 | 0.07 |
|  |  | Lagunas (Lagoons) | 10,020 | 0.16 |
|  |  | Salares (Salt) | 6,477 | 0.10 |
| Total |  |  | 6,331,644 | 99.98 |

# 3. SWAT Equivalent Land Cover

SWAT requires 44 parameters for each land cover type, which are shown in Appendix 1 and described at https://swat.tamu.edu/media/69341/ch14_input_plantdb.pdf. The database distributed with the SWAT model includes these parameters for the land cover types listed in Appendix 2. Therefore, the SWAT land cover database was used as starting point for local land cover database development. This section explains the process of selecting the SWAT equivalent to local classes at Level III.  The process included reviewing images, descriptions and consulting with experts in the region as explained in Sections 3.1 to 3.7. The identified equivalent land cover categories based on the terminology of the SWAT land cover database are shown in Table 2, column 3.

Table 2. SWAT equivalent land covers that serve as the basis for land cover parameters in the Arequipa Region (First two columns are repeated from Table 1) (English translations in parentheses)

| Level II | Level III | Equivalent land cover (SWAT name) (The basis for selecting these is described in sections 3.1 to 3.7). |
| --- | --- | --- |
| Cultivos Permanentes (Permanent crops) | Cultivos Agrícolas (Agricultural crops) | Agricultural Land-Generic (AGRL) |
| Áreas Urbanizadas (Urbanized Areas) | Zonas Urbanas (Urban areas) | Residential-High Density (URHD) |
| Vegetación Asociada a Cuerpos de Agua (Water associated vegetation) | Bofedales (Peat bogs) | Wetlands-Non-Forested (WETN) |
|  | Humedal Costero (Coastal wetland) | Wetlands-Non-Forested (WETN) |
|  | Monte Ribereño (Floodplain) | Wetlands-Forested (WETF) |
| Áreas con Vegetación Herbácea y/o Arbustiva (Areas with herbaceous and/or shrubby vegetation) | Césped de Puna (Puna grass) | Shrubland (SHRB) |
|  | Lomas (Fog-watered vegetation) | Grassland (GRAS) |
|  | Matorral (Scrub) | Mixed Grassland/Shrubland (MIGS) |
|  | Pajonal (Scrubland) | Grassland (GRAS) |
|  | Tillandsia (Evergreen flowering plants) | Barren or Sparsely Vegetated (BSVG) |
|  | Tolares (Resinous shrub) | Shrubland (SHRB) |
|  | Yaretales (High altitude shrubs) | Shrubland (SHRB) |
| Áreas Sin o Con Poca Vegetación (Areas without or with little vegetation) | Áreas con Vegetación Herbácea Veraniegas (Areas with summer herbaceous vegetation) | Herbaceous Tundra (TUHB) |
|  | Sin Vegetación (No vegetation) | Barren (BARR) |
|  | Vegetación de Suelos Crioturbados (Frozen soil vegetation) | Barren or Sparsely Vegetated (BSVG) |
| Bosques (Forest) | Bosque de Lomas (Fog-watered forest) | Forest-Evergreen (FRSE) |
|  | Queñuales (Polylepis) | Forest-Evergreen (FRSE) |
|  | Cactaceas (Cactus) | Barren or Sparsely Vegetated (BSVG) |
|  | Puyas (Giant bromeliad) | Mixed Grassland/Shrubland (MIGS) |
| Aguas continentales (Waterbodies) | Embalses (Reservoir) | Water (WATR) |
|  | Lagunas (Lagoons) | Water (WATR) |
|  | Salares (Salt) | Water (WATR) |

## 3.1. Cultivos Permanentes (Permanent crops)

3.1.1. Cultivos Agrícolas (Agricultural crops)

SWAT has the following generic options to simulate agricultural lands; in addition, users can specify specific crops as listed in Appendix 2:

- Agricultural Land-Generic (AGRL)

- Agricultural Land-Row Crops (AGRR)

- Agricultural Land-Close-grown (AGRC)

Based on the land cover map of Arequipa, 2.39% of the region is covered by agricultural crop.  Field sizes are very small, however and the variety of crops grown is quite diverse. (Figure 2). Therefore, Agricultural Land-Generic (AGRL) was used as the base land cover category for agricultural lands in Arequipa Region.

Specific crops (listed in Appendix 2) can be used, if users want to model specific region and have more detail about its crop types.

Figure 2. Agricultural Lands in Arequipa Region, derived from the land cover map in Figure 1

## 3.2.  Áreas Urbanizadas (Urbanized Areas)

3.2.1. Zonas Urbanas (Urban areas)

SWAT has the following options to simulate Urban lands, depending on the ratio of impervious land and fraction of directly connected impervious land:

- Residential (URBN)

- Residential-High Density (URHD)

- Residential-Medium Density (URMD)

- Residential-Med/Low Density (URML)

- Residential-Low Density (URLD)

- Commercial (UCOM)

- Industrial (UIDU)

- Transportation (UTRN)

- Institutional (UINS)

Urban lands are mostly associated with the city of Arequipa (Figure 3), which is residential with high density. Therefore, Residential-High Density (URHD) was used in general for the entire region. However, depending of the study area, other urban lands can be used.

Figure 3. Urban lands in Arequipa Region, derived from the land cover map in Figure 1

## 3.3. Vegetación Asociada a Cuerpos de Agua (Vegetation associated with water)

3.3.1. Bofedales (Peat bogs)

The term “Bofedales” refers to high altitude wetland with underlying peat layers [3]. These small wetlands are humid and are considered as small native prairie [4,5]. Bofedales are traditionally used as sources of forage and water supply for livestock in high altitudes of Peru [3].

Figure 4. Bofedales in Callali, Arequipa [6]

SWAT has the following options for Wetlands:

- Wetlands-Non-Forested (WETN)

- Wetlands-Forested (WETF)

- Wetlands-Mixed (WETL)

Based on Bofedales’s definition and images, Wetland-Non-Forested (WETN) was used as the SWAT equivalent base land cover category.

3.3.2. Humedal Costero (Coastal wetland)

Coastal wetlands form along the arid Pacific Coast of Peru around shallow bays, coastal lagoons, river mouths and estuaries (see Figure 5) [7]. Similar to Bofedales, Wetland-Non-Forested (WETN) was also used as the base land cover category for Humedal Costero.

Figure 5. Humedal Costero in Chile [8]

3.3.3. Monte Ribereño (Floodplain)

Monte Ribereño is translated as riparian mountains in English and refers to dense biomes (mostly trees and shrubs) grown on the banks of rivers and other waterways. Therefore,  Wetlands-Forested (WETF) was used as the base land cover category.[9].

Figure 6. Monte Ribereno [9]

## 3.4. Áreas con Vegetación Herbácea y/o Arbustiva (Areas with herbaceous and/or shrubby vegetation)

3.4.1. Césped de Puna (Puna grass)

Puna refers to the montane grassland and shrubland biome that is found in the central Andes Mountains of South America, including Peru [10]. There are two types of puna: wet/moist puna and dry puna. Southern regions of Peru are mostly covered by central Andean dry puna, dominated by shrublands and thickets of tola shrubs [10]. Therefore, Shrubland (SHRB) was used as its base SWAT equivalent land cover.

Figure 7. Cesped de Puna [11]

3.4.2. Lomas (Fog-watered vegetation)

Loma is the Spanish term for “hills”, and refers to fog-watered pockets of vegetation in the coastal desert of Peru and northern Chile [12,13]. This isolated vegetation in arid deserts exists because of the moist dense fog and mist which rolls in from the Pacific [12]. Grassland (GRAS) was used as its base land cover category.

Figure 8. Lomas in protected area north of Lima [12]

3.4.3. Matorral (Scrub)

Matorral is a Spanish word for shrubland, thicket or bushes [14]. It refers to vegetated land, mostly covered by shrubs or bushes, and often includes grasses, herbaceous plants and geophytes [15]. Therefore, Mixed Grassland/Shrubland (MIGS) was used as its base land cover.

Figure 9. Springtime in Chilean matorral a few kilometers north of Santiago [14]

3.4.4. Pajonal (Scrubland)

Pajonal is characterized by dense clusters of tall grasses with hard leaves, known by the common name of ichus, whose species predominant are Stipa sp., Festuca orthophylla, Stipa ichu, Stipa feathery, etc. The plant stage is constituted by an abundant mixture of grasses and other herbs of perennial habitat [1]. Grassland (GRAS) was used as its base SWAT equivalent.

| Figure 10. Pajonal in Bolivia [16] | Figure 11. Pajonal [17] |
| --- | --- |

3.4.5. Tillandsia (Evergreen flowering plants)

Tillandsia refers to group of perennial flowering plants in the family Bromeliaceae with roughly 650 species [18]. Barren or Sparsely Vegetated (BSVG) was used as its SWAT equivalent.

Figure 12. Tillandsia [19]

3.4.6. Tolares (Resinous shrub)

Tolares refers to open herbaceous formations with a mixture of woody plants, among which Parastrephia lucida and Baccharis tricuneata called “tola” stand out, which have been used as fuel, especially by bakeries in the city of Arequipa [1]. Shrubland (SHRB) was used as its SWAT equivalent land cover.

Figure 13.Tolares (Figure 43 of the national plant cover map of Peru document [20])

3.4.7. Yaretales

Yaretales refers to shrubs that grow in high altitude arid regions of the Andes mountains. [1]. Similar to Tolares, Shrubland (SHRB) was used as SWAT equivalent land cover for Yaretales.

Figure 14. Peru Arequipa Colca Reserva National Salinas Yaretales [21]

## 3.5. Áreas Sin o Con Poca Vegetación (Areas without or with little vegetation)

3.5.1. Áreas con Vegetación Herbácea Veraniegas (Areas with summer herbaceous vegetation)

Based on its explanatory name, Herbaceous Tundra (TUHB) was used as its SWAT equivalent.

3.5.2. Sin Vegetación (No vegetation)

It refers to bare lands (either in high or low altitudes), where no plant exists naturally [1]. Therefore, Barren (BARR) was used as its SWAT equivalent .

3.5.3. Vegetación de Suelos Crioturbados (Frozen soil vegetation)

It refers to vegetation at high altitude (extending 5,000 meter above the sea level) lands, that are covered with rocks and perennial snow on the ground. However, some Lichens can be observed on the rocks when they are not covered with snow. This vegetation might exist in several colors of yellow, orange, green or blue. It also includes some rose plants that grow flush [1]. Barren or Sparsely Vegetated (BSVG) was used as the base land cover.

## 3.6. Bosques (Forest)

3.6.1. Bosque de Lomas (Fog-watered forest)

Similar to the Lomas, these are areas where coastal fog is sufficient to support pockets of forested vegetation with the coastal desert.  Based on its explanatory name, Forest-Evergreen (FRSE) was used as its SWAT Equivalent.

3.6.2. Queñuales (Polylepis)

It was translated as “Polylepis” in English and refers to complexes of native vegetation (including 28 shrub and tree species) in the mid, and high altitudes of the tropical Andes [22]. Therefore, Forest-Evergreen (FRSE) was used as its SWAT equivalent.

Figure 15. Queñual in Puna, Peru [23]

3.6.3. Cactaceas (Cactus)

Barren or Sparsely Vegetated (BSVG) was used as SWAT equivalent for cactus.

3.6.4. Puyas

Puya is a type of the botanical family Bromeliaceae, subfamily Pitcairnioideae [24]. Mixed Grassland/Shrubland (MIGS) was used as its SWAT equivalent.

Figure 16. Puya [25]

## 3.7. Aguas continentals (Waterbodies)

There are three level III waterbodies in the ZEE land cover classification for Arequipa including: Embalses (Reservoir), Lagunas (Lagoons), and Salar (Salt).

SWAT has only one land cover for water (named WATR in database). It was used for all three forms of waterbodies in the Arequipa Region.

# 4. Regionalization of Land Cover Parameters

The SWAT default parameter values of selected equivalent land covers were used as the starting point for newly developed local land cover classes. Then parameters that control the seasonal growth cycle were adjusted to better reflect the characteristics of local land cover complexes. SWAT uses two sets of plant growth models depending on whether plants are seasonal or perennial [26]. Most of the land cover in the Arequipa Region is perennial, plants maintain their roots throughout the year, becoming dormant in the winter months. (See the description of how these are simulated differently in the SWAT Plant Database [27]). Therefore, three parameters that represent the seasonal growth cycle of perennial crops were updated, including: 1) minimum leaf area index (LAI) for the plant during the dormant period (ALAI_MIN), 2) maximum leaf area index (BLAI), and 3) optimal temperature for plant growth (T_OPT). Also for perennial land covers, SWAT allows roots to grow up to the maximum rooting depth provided for the soil and land cover, therefore modification of rooting depth was not needed.

The rest of this section explains the process for estimation of these three parameters and example results for Pajonal.

## 4.1. Leaf Area Index Parameters

The two LAI parameters, ALAI_MIN and BLAI, were estimated based on remote sensing for each land cover in four steps as shown in Figure 17:

- Several locations where the land cover was dominant were selected (Figure 17-a)

- LAI timeseries (4-day, 500 m resolution) for those locations were obtained from the Moderate Resolution Imaging Spectroradiometer (MODIS)/ Visible Infrared Imaging Radiometer Suite (VIIRS) subsets [28] (Figure 17-b).

- LAI were averaged for several years (e.g. 2010-2017) to obtain an average annual LAI cycle (Figure 17-c).

- Minimum LAI during dormant period (ALAI_MIN) and maximum LAI (BLAI) were obtained from the average LAI cycle as shown in Figure 17-c.

Figure 17. The process to obtain BLAI and ALAI_MIN for Pajonal in Arequipa Region. (a) selected locations dominated by pajonal, (b) MODIS LAI timeseries of selected locations, and (c) average annual LAI cycle and estimated LAI parameters for pajonal

## 4.2. Optimal Temperature for Plant Growth

The optimal temperature for plant growth (T_OPT) was assumed to be the temperature where each land cover exists. Therefore, T_OPT was estimated in two steps based on the average annual temperature [29] of locations covered by each land cover, as shown in Figures 18 and 19:

- The land cover map (Figure 18-a) was overlaid with the 30-years (1988-2017) average annual temperature map of Arequipa [29] (Figure 18-b) to obtain average temperature of pixels associated with each land cover (Figure 18-c).

- The average temperature of overlaid pixels (Figure 19) was obtained and used as optimal temperature of given land cover.

Figure 18. The process to obtain average temperature map of Pajonal

Figure 19. Screenshot of Pajonal temperature variation in the region and highlighted average temperature

# 5. Sample Results

The final result was a set of new local land covers. Figure 20 shows parameters in the SWAT database for pajonal (named “PJNL”) as an example. Highlighted parameters were modified (as described in section 4), while the rest of parameters were populated by values for Grassland (GRAS) that was used as its base SWAT equivalent.

Figure 20. Pajonal parameters in the SWAT database. highlighted parameters were modified to reflect seasonal growth cycle

This methodology was used for case study of El Frayle watershed in upstream of Chili River (Figure 21). Table 3 shows coverage of new classes in the watershed and values that were used for ALAI_MIN, BLAI, and T_OPT. modified parameters for four local vegetation are shown in bold.

Figure 21. Land cover map of El Frayle watershed

Table 3. New land covers and associated seasonal cycle parameters. Parameters that were adjusted for local vegetation are shown in bold.

| Local name | Area (%) | New SWAT land cover | SWAT original land cover class | Adjusted land cover parameters* |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ALAI_MIN | BLAI | T_OPT (°C) |
| Bofedales | 4.7 | BFDL | Wetlands-Non-Forested | 0 | 6 | 25 |
| Césped de Puna | 3.9 | CDPN | Shrubland | 0.2 | 1.5 | 3.6 |
| Pajonal | 45.1 | PJNL | Grassland | 0.1 | 0.3 | 3.45 |
| Tolares | 33.2 | TLRS | Shrubland | 0.1 | 0.75 | 3.85 |
| Yaretales | 0.8 | YRTL | Shrubland | 0.1 | 0.2 | 0.35 |
| Sin Vegetación | 10.6 | SNVG | Barren | 0 | 0.01 | 25 |
| Vegetación de Suelos Crioturbados | 1.7 | VDSC | Barren or sparsely vegetated | 0 | 1.5 | 25 |

*ALAI_MIN: minimum leaf area index during the dormant period; BLAI: maximum leaf area index; T_OPT: optimal temperature for plant growth

# 6. References:

[1] Gobierno Regional Arequipa, 2016. Elaboración del Estudio de Suelos y la Clasificación de Tierras por su Capacidad de Uso Mayor en la Región Arequipa. Proyecto Desarrollo de Capacidades en Zonificación Ecológica Económica para el Ordenamiento Territorial en la Región Arequipa, Autoridad Regional Ambiental de Arequipa, Gobierno Regional Arequipa, Arequipa, Perú. 1,152 pp. (Reg.# 1782331, Doc.# 1775929).

[2] Arnold, J.G., Kiniry, J.R., Srinivasan, R., Williams, J.R., Haney, E.B. and Neitsch, S.L., 2013. SWAT 2012 input/output documentation. Texas Water Resources Institute. URL: https://swat.tamu.edu/media/69296/swat-io-documentation-2012.pdf

[3] Fonkén, M.M., 2014. An introduction to the bofedales of the Peruvian High Andes. Mires and Peat, 15(5).

[4] Bofedals, Wikipedia, URL: https://es.wikipedia.org/wiki/Bofedal

[5] The shape of things to come.URL: https://shapeofthingstocome.org/2014/09/19/what-the-hell-is-a-bofedal/

[6] Bofedal in Callali, Arequipa, Peru. URL: https://www.flickr.com/photos/pghernandez/39540673935/in/photostream/

[7] Tabilo E., Burmeister J., Chávez-Villavicencio C. and Zöckler C. Chain of Coastal Wetlands at the Arid Pacific Coast of South America. URL: http://www.unesco.org/new/fileadmin/MULTIMEDIA/HQ/SC/images/CoastalWetlands_PeruChile_vs2.pdf

[8] Humedal Costero, Laguna Conchalí, URL: https://www.flickr.com/photos/131191727@N04/26536152738

[9] Monte Ribereño, URL: http://biologiaenel30.blogspot.com/2014/04/monte-ribereno.html

[10] Puna grassland, Wikipedia, URL: https://en.wikipedia.org/wiki/Puna_grassland

[11] Puna, Peu. URL: https://www.flickr.com/photos/elsasalinas/16879408770

[12] Lomas, Wikipedia, URL: https://en.wikipedia.org/wiki/Lomas

[13] Lomas, The Nature Conservancy, URL: https://www.nature.org/en-us/about-us/where-we-work/latin-america/peru/lomas-de-atiquipa/

[14] Matorral, Wikipedia, URL: https://en.wikipedia.org/wiki/Matorral

[15] Matorral, Spanish Wikipedia, URL: https://es.wikipedia.org/wiki/Matorral

[16] Pajonal, Wikipedia, URL: https://es.m.wikipedia.org/wiki/Archivo:Pajonal_con_la_Laguna_Verde_al_fondo_(Potos%C3%AD_-_Bolivia).jpg

[17] Pixabay, URL: https://pixabay.com/es/photos/pajonal-paisaje-aventura-hierba-3541116/

[18] Tillandsia, Wikipedia, URL: https://en.wikipedia.org/wiki/Tillandsia

[19] Tillandsia purpurea. URL: https://commons.wikimedia.org/wiki/File:Tillandsia_purpurea_(Pfendbach)_00433.jpg

[20] Mapa nacional de cobertura vegetal, Ministerio del Ambiente, URL: http://www.minam.gob.pe/patrimonio-natural/wp-content/uploads/sites/6/2013/10/MAPA-NACIONAL-DE-COBERTURA-VEGETAL-FINAL.compressed.pdf

[21] Peru Arequipa Colca Reserva National Salinas Yaretales, URL: http://www.tripsanthology.com/canyon-de-colca-vertigo-and-beauty/peru-arequipa-colca-reserva-national-salinas-yaretales/

[22] Polylepis, Wikipedia, URL: https://en.wikipedia.org/wiki/Polylepis

[23] Queñual (Polylepis incana), URL: https://www.flickr.com/photos/eduardodios/16943323508

[24] Puya (plant), Wikipedia, URL: https://en.wikipedia.org/wiki/Puya_(plant)

[25] Puya Raimondii, URL: https://commons.wikimedia.org/wiki/File:Puya_Raimondii.JPG

[26] Neitsch, S. L., Arnold, J. G., Kiniry, J. R., Williams, J. R. 2011. Soil and water assessment tool theoretical documentation version 2009. Texas Water Resources Institute. URL: https://swat.tamu.edu/media/99192/swat2009-theory.pdf

[27] Arnold, J.G., Kiniry, J.R., Srinivasan, R., Williams, J.R., Haney, E.B. and Neitsch, S.L., 2013. SWAT 2012 input/output documentation. Appendix A. Texas Water Resources Institute. URL: https://swat.tamu.edu/media/69419/Appendix-A.pdf

[28] Myneni, R., Knyazikhin, Y., Park, T., 2015. MCD15A2H MODIS/Terra+Aqua Leaf Area Index/FPAR 8-day L4 Global 500m SIN Grid V006. NASA EOSDIS Land Processes DAAC. http://doi.org/10.5067/MODIS/MCD15A2H.006

[29] Moraes, A. G. L., Cherkauer, K. A., Bowling, L. C., Zeballos Velarde, C. R., 2019. Arequipa Climate Maps. Purdue University Research Repository. doi:10.4231/490D-HC66

# Appendix 1. Land Cover/Plant Growth parameters needed for SWAT

Figure A1. Screenshot of SWAT parameters used to control the annual growth and water use for land cover types.

Table Al. Parameters in the land cover/plant growth database [2], listed in the order they are shown in Figure A1.

| Parameter | Min | Max | Definition |
| --- | --- | --- | --- |
| Crop Type Parameters |  |  |  |
| Crop Name |  |  | The name of the land cover. (Note, not all are crops; they include barren and natural lands.) The 135 choices are listed in Appendix 2. |
| CPNM |  |  | 4-letter code for each Crop Name |
| IDC |  |  | Categories of plants: Warm season annual legume, Cold season annual legume, Perennial legume, Warm season annual, Cold season annual, Perennial, or Trees |
| Op Schedule |  |  | 4-letter code for management practices operations |
| BIO_E | 10 | 90 | Biomass/Energy Ratio. |
| HVSTI | 0.01 | 1.25 | Harvest index. |
| BLAI | 0.5 | 10 | Max leaf area index. |
| FRGRW1 | 0 | 1 | Fraction of the plant growing season corresponding to the 1st. Point on the optimal leaf area development curve. |
| LAIMX1 | 0 | 1 | Fraction of the max. leaf area index corresponding to the 1st. point on the optimal leaf area development curve. |
| FRGRW2 | 0 | 1 | Fraction of the plant growing season corresponding to the 2nd. point on the optimal leaf area development curve. |
| LAIMX2 | 0 | 1 | Fraction of the max. leaf area index corresponding to the 2nd. point on the optimal leaf area development curve. |
| DLAI | 0.15 | 1 | Fraction of growing season when leaf area starts declining. |
| CHTMX | 0.1 | 20 | Max canopy height. |
| RDMX | 0 | 3 | Max root depth. |
| T_OPT | 11 | 38 | Optimal temp for plant growth. |
| T_BASE | 0 | 18 | Min temp plant growth. |
| CNYLD | 0.0015 | 0.075 | Fraction of nitrogen in seed. |
| CPYLD | 0.0001 | 0.015 | Fraction of phosphorus in seed. |
| BN1 | 0.004 | 0.07 | Fraction of N in plant at emergence. |
| BN2 | 0.002 | 0.05 | Fraction of N in plant at 0.5 maturity. |
| BN3 | 0.001 | 0.27 | Fraction of N in plant at maturity. |
| BP1 | 0.0005 | 0.01 | Fraction of P at emergence. |
| BP2 | 0.0002 | 0.007 | Fraction of P at 0.5 maturity. |
| BP3 | 0.0003 | 0.0035 | Fraction of P at maturity. |
| WSYF | -0.2 | 1.1 | Lower limit of harvest index. |
| USLE_C | 0.001 | 0.5 | Min value of USLE C factor applicable to the land cover/plant. |
| GSI | 0 | 5 | Max stomatal conductance (in drought condition). |
| VPDFR | 1.5 | 6 | Vapor pressure deficit corresponding to the fraction maximum stomatal conductance defined by FRGMAX |
| FRGMAX | 0 | 1 | Fraction of maximum stomatal conductance that is achievable at a high vapor pressure deficit. |
| WAVP | 0 | 50 | Rate of decline in radiation use efficiency per unit increase in vapor pressure deficit. |
| CO2HI | 300 | 1000 | Elevated CO2 atmospheric concentration. |
| BIOEHI | 5 | 100 | Biomass-energy ratio corresponding to the 2nd. point on the radiation use efficiency curve. |
| RSDCO_PL | 0.01 | 0.099 | Plant residue decomposition coefficient. |
| ALAI_MIN | 0 | 0.99 | Minimum leaf area index for plant during dormant period |
| BIO_LEAF | 0 | 1 | Fraction of tree biomass converted to residue during dormancy |
| MAT_YRS | 0 | 100 | Number of years required for tree species to reach full development |
| BMX_TREES | 0 | 5000 | Maximum biomass for a forest |
| EXT_COEF | 0 | 2 | Light extinction coefficient |
| BM_DIEOFF | 0 | 1 | Biomass die-off fraction |
| Hydrological parameters |  |  |  |
| OV_N | 0.01 | 30 | Manning's "n" value for overland flow. |
| CN2A | 25 | 98 | SCS runoff curve number for moisture condition II. |
| CN2B | 25 | 98 | SCS runoff curve number for moisture condition II. |
| CN2C | 25 | 98 | SCS runoff curve number for moisture condition II. |
| CN2D | 25 | 98 | SCS runoff curve number for moisture condition II. |

# Appendix 2. List of crops that are available in SWAT database [2]

| - Agricultural Land-Close-Grown | - Forest-Mixed | - Red Clover |
| --- | --- | --- |
| - Agricultural Land-Generic | - Garden or Canning Peas | - Rice |
| - Agricultural Land-Row Crops | - Grain Sorghum | - Rubber Trees |
| - Alamo Switchgrass | - Grarigue | - Russian Wildrye |
| - Alfalfa | - Grassland | - Rye |
| - Almonds | - Green Beans | - Savanna |
| - Alsike Clover | - Hay | - Septic Area |
| - Altai Wildrye | - Head Lettuce | - Sesbania |
| - Apple | - Herbaceous Tundra | - Shrubland |
| - Asparagus | - Herbaceous Wetland | - Sideoats Grama |
| - Bananas | - Honey Mesquite | - Slender Wheatgrass |
| - Bare Ground Tundra | - Honeydew Melon | - Smooth Bromegrass |
| - Barren or Sparsely Vegetated | - Indiangrass | - Snow or Ice |
| - Barren | - Irrigated Cropland And Pasture | - Sorghum Hay |
| - Bell Pepper | - Italian (Annual) Ryegrass | - Southwestern Us (Arid) Range |
| - Bermudagrass | - Johnsongrass | - Soybean |
| - Big Bluestem | - Kentucky Bluegrass | - Spinach |
| - Broccoli | - Lentils | - Spring Barley |
| - Cabbage | - Lima Beans | - Spring Canola-Argentine |
| - Cantaloupe | - Little Bluestem | - Spring Canola-Polish |
| - Carrot | - Maple | - Spring Wheat |
| - Cashews | - Meadow Bromegrass | - Strawberry |
| - Cassava | - Mint | - Sugarbeet |
| - Cauliflower | - Mixed Dryland/Irrigated Cropland | - Sugarcane |
| - Celery | - Mixed Forest | - Summer Pasture |
| - Cockle Burr | - Mixed Grassland/Shrubland | - Sunflower |
| - Cocoa Tree | - Mixed Tundra | - Sweet Corn |
| - Coconut | - Mung Beans | - Sweetclover |
| - Coffee | - Oak | - Sweetpotato |
| - Corn | - Oats | - Tall Fescue |
| - Corn Silage | - Oil Palm | - Timothy |
| - Cowpeas | - Olives | - Tobacco |
| - Crested Wheatgrass | - Onion | - Tomato |
| - Cropland/Grassland Mosaic | - Orange | - Upland Cotton-Harvested With |
| - Cropland/Woodland Mosaic | - Orchard | - Upland Cotton-Harvested With |
| - Cucumber | - Papayas | - Vineyard |
| - Deciduous Broadleaf Forest | - Parthenium | - Walnut |
| - Deciduous Needleleaf Forest | - Pasture | - Water |
| - Dryland Cropland And Pasture | - Peanut | - Water Bodies |
| - Durum Wheat | - Pearl Millet | - Watermelon |
| - Eastern Gamagrass | - Peppers | - Western Wheatgrass |
| - Eggplant | - Pine | - Wetlands-Forested |
| - Eragrostis Teff | - Pineapple | - Wetlands-Mixed |
| - Eucalyptus | - Pinto Beans | - Wetlands-Non-Forested |
| - Evergreen Broadleaf Forest | - Plaintains | - Willow |
| - Evergreen Needleleaf Forest | - Poplar | - Winter Barley |
| - Field Peas | - Potato | - Winter Pasture |
| - Flax | - Radish | - Winter Wheat |
| - Forest-Deciduous | - Range-Brush | - Wooded Tundra |
| - Forest-Evergreen | - Range-Grasses | - Wooded Wetland |